# IMU_library README

## Overview

IMU_library is a CircuitPython library for working with the Adafruit LSM6DS3 IMU (3-axis accelerometer + gyroscope). It provides:

- Quaternion-based complementary filter for fusing accelerometer and gyroscope data.

- Euler angle outputs (roll, pitch, yaw) and angular rates on demand.

- High update rate optimizations for CircuitPython.

- Runtime calibration of sensor offsets, with optional hard-coded offsets.

- Adjustable filter weight (alpha) and accelerometer/gyro output data rates (ODR).

## Limitations

- Yaw drift: Without a magnetometer, yaw will drift over time because the accelerometer cannot correct rotation about the vertical axis.

- Sensor offsets: Raw gyro and accelerometer readings may have small biases; calibration is recommended for best results.

- Update rate dependency: For accurate orientation tracking, update() should be called at a consistent high frequency, ideally near the sensor ODR (e.g., 100 Hz).

## Dependencies

- board (CircuitPython board pin definitions)

- time (for monotonic timing)

- math (trigonometry)

- adafruit_lsm6ds.lsm6ds3

- adafruit_lsm6ds (for Rate constants)

Make sure these libraries are installed on your CircuitPython device.

## Installation

- Copy IMU_library.py to your CircuitPython device (e.g., to the /lib/ folder or same folder as your main script).

- Import the library in your program:

from IMU_library import IMU

## Basic Usage

import time
from IMU_library import IMU

# Initialize IMU (uses default I2C)
imu = IMU()

# Set desired update rate for IMU calculations (e.g., 100Hz)
desired_update_hz = 100
update_interval = 1.0 / desired_update_hz
last_update_time = time.monotonic()

while True:
    current_time = time.monotonic()
    
    if current_time - last_update_time >= update_interval:
        last_update_time += update_interval
        imu.update()
        
        # Access angles and angular rates
        roll, pitch, yaw = imu.angle_estimates
        gx, gy, gz = imu.angular_rates
        print("Roll:", roll, "Pitch:", pitch, "Yaw:", yaw)
        print("gx:", gx, "gy:", gy, "gz:", gz)

## Calibration

- Runtime calibration: Call imu.calibrate(duration=2.0) while the sensor is stationary and flat. This computes gyro and accelerometer offsets and applies them internally.

- Hard-coded offsets: After calibration, you can store the values and set them manually using:

imu.set_offsets(gyro_offset=(gx_off, gy_off, gz_off),
                accel_offset=(ax_off, ay_off, az_off))

- Recommended for repeated use to avoid having to recalibrate every time.

## Advanced Usage

- Adjust filter weight (alpha): Higher values favor gyro, lower values favor accelerometer.

imu.alpha = 0.97

- Change sensor data rates at runtime:

from adafruit_lsm6ds import Rate
imu.accel_rate = Rate.RATE_104_HZ
imu.gyro_rate = Rate.RATE_104_HZ

- Throttle update() using a monotonic timer to maintain a stable integration rate, independent of printing or other loop tasks.

## Example Programs

- Advanced Example: Demonstrates calibration, dynamic alpha/ODR changes, separate IMU update and control loops, and periodic output.

  - File: example_code_for_IMU_library_detailed.py

  - Good for students or applications needing fine control.

- Simple Example: Minimal setup showing only initialization, updates, and printing angles/gyro rates.

  - File: example_code_for_IMU_library_simple.py

  - Ideal for beginners or quick testing.

## Notes

- Euler Angles: Roll and pitch are corrected using the accelerometer; yaw is uncorrected and will drift.

- Update Rate: For consistent integration, call update() at a rate near the sensor ODR (e.g., 100 Hz for default 104 Hz ODR).

- Performance: Avoid printing inside the high-frequency loop; print at a slower rate (1–5 Hz) to prevent slowing down integration.

## Quick Reference Table

| Method / Property | Description |
| --- | --- |
| imu.update() | Update the quaternion and filter integration. Call frequently (~100 Hz). |
| imu.angle_estimates | Returns (roll, pitch, yaw) in degrees. Access only after update(). |
| imu.angular_rates | Returns (gx, gy, gz) in degrees per second (corrected for offsets). |
| imu.update_rate_hz | Returns the average IMU update rate in Hz. |
| imu.calibrate(duration) | Calibrates gyro and accelerometer offsets over duration seconds. |
| imu.set_offsets(gyro_offset, accel_offset) | Set previously measured offsets manually. |
| imu.alpha | Complementary filter weight; adjust gyro vs accel trust. |
| imu.accel_rate | Accelerometer output data rate (ODR); can be changed at runtime. |
| imu.gyro_rate | Gyroscope output data rate (ODR); can be changed at runtime. |